Claude

| Parameter | Value |
| --- | --- |
| Input voltage (LiPo 2S battery) | VIN = 7.4 V |
| Output voltage | VOUT = 3.3 V |
| Nominal output current | IS = 200 mA |
| Inductor current ripple | ΔIL = 30% of IS |
| Max output voltage ripple | ΔVS = 50 mV |
| Switching frequency (LT1959) | f = 500 kHz |

# 1. Duty Cycle

For an ideal buck converter in continuous mode:

D = VOUT / VIN

D = 3,3 / 7,4

D = 0,446   →   ton = 0,892 µs   /   toff = 1,108 µs

Check : D = 44,6 % < 50 % → peak current IP = 4,5 A guaranteed by LT1959.

# 2. Inductor L1

## 2.1 Target current ripple

The peak-to-peak current ripple in the inductor is set to 30% of the output current:

ΔIL = 0,30 × IS = 0,30 × 200 mA

ΔIL = 60 mA

## 2.2 Minimum value of L

The formula from the datasheet LT1959 for continuous conduction mode is:

L = (VOUT × (VIN - VOUT)) / (VIN × ΔIL × f)

L = (3,3 × (7,4 - 3,3)) / (7,4 × 0,060 × 500×10³)

L = (3,3 × 4,1) / (7,4 × 30 000)

L = 13,53 / 222 000

L_min = 60,9 µH

Standard value: L1 = 10 µH (standard serie, available with Isat > 500 mA).

Note: with L = 10 µH, the actual ripple will be lower than the 30% target:

ΔIL_réel = (VOUT × (VIN - VOUT)) / (VIN × L × f)

ΔIL_réel = (3,3 × 4,1) / (7,4 × 10×10⁻⁶ × 500×10³)

ΔIL_real ≈ 365 mA  →  18 % ok IS  ✓ (< 30 %)

# 3. peak and valley currents in the inductor

IL_peak   = IS + ΔIL_real/2 = 200 + 183  = 383 mA

IL_valley = IS - ΔIL_real/2 = 200 - 183  =  17 mA

Continuous conduction mode check: IL_valley > 0  ✓

Saturation current required: Isat > IL_peak = 383 mA → inductor standard 10 µH ok.

# 4. Output capacitor C3

## 4.1 ESR constraint (voltage ripple)

The output voltage ripple is mainly determined by the capacitor’s ESR:

ΔVS ≈ ΔIL_real × ESR

ESR_max = ΔVS / ΔIL_real = 50×10⁻³ / 0,365

ESR_max = 136 mΩ

Chosen capacitor: AVX TPS 100 µF / 10 V (tantalum solide, ESR typ. 100 mΩ < 136 mΩ)  ✓

## 4.2 Capacitance value

The value in µF is not critical. With C3 = 100 µF:

Capacitive ripple = ΔIL / (8 × f × C3)

= 0,365 / (8 × 500×10³ × 100×10⁻⁶)

= 0,91 mV (negligible compared to the ESR contribution)

# 5. Input capacitor C1

The RMS current in the input capacitor is:

I_C1_rms = IS × √(VOUT × (VIN - VOUT)) / VIN

I_C1_rms = 0,200 × √(3,3 × 4,1 / 7,4)

I_C1_rms = 0,200 × √(0,183)

I_C1_rms ≈ 85,5 mA

Chosen capacitor: C1 = 100 µF / 10 V, tantalum AVX TPS (Iripple ≥ 0,7 A)  ✓

# 6. Freewheel diode D1

Average current in D1 :

ID1_avg = IS × (VIN - VOUT) / VIN

ID1_avg = 0,200 × (7,4 - 3,3) / 7,4

ID1_avg = 110 mA

Max reverse voltage = VIN = 7,4 V

Selected Diode: MBRS330T3 (Schottky, 3 A / 30 V, VF typ. 0,5 V @ 3 A)  ✓

# 7. Capacitor BOOST C2

The capacitor BOOST supplies the switch NPN driver to saturate it. The minimum value is:

C2_min = (IS/50) / (f × 0,6)

C2_min = (0,200/50) / (500×10³ × 0,6)

C2_min ≈ 13 nF  →  C2 selected = 0,68 µF (datasheet standard value)  ✓

Diode D2 : 1N914 or 1N4148, anode connected to VOUT.

# 8. Feedback network (R1, R2)

The internal voltage is VREF = 1,21 V. The resistive divider R1/R2 fixes VOUT :

VOUT = VREF × (1 + R1/R2)

→  R1/R2 = (VOUT/VREF) - 1 = (3,3/1,21) - 1 = 1,727

Choice of R2 ≤ 2,5 kΩ (datasheet recommandation) : R2 = 2,49 kΩ

R1 = R2 × (VOUT/VREF - 1) = 2490 × 1,727

R1 = 4 298 Ω  →  standard value: R1 = 4,32 kΩ (E96, 1 %)

Verification:

VOUT = 1,21 × (1 + 4320/2490) = 1,21 × 2,735

VOUT = 3,309 V  ✓  (error < 0,3 %)

# 9. Compensation capacitor VC (CC)

The default compensation recommended by the datasheet for most applications is:

CC = 1,5 nF  (condensateur seul, RC = 0)

This network creates a pole at:

f_pole = 1 / (2π × ROUT_EA × CC) ≈ 530 Hz

The loop phase margin remains > 60° at the crossover frequency (≈ 100 kHz), ensuring stability.

# 10. Components summary

| Reference | Value / Type | Role | Design criterion |
| --- | --- | --- | --- |
| L1 | 10 µH, Isat > 400 mA | Filtering inductor | ΔIL ≈ 18 % |
| C1 | 100 µF / 10 V Tant. | Input capacitor | Iripple ≥ 86 mA |
| C2 | 0,68 µF ceramic | Capacitor BOOST | Charge driver NPN |
| C3 | 100 µF / 10 V Tant. | Output capacitor | ESR ≤ 136 mΩ |
| CC | 1,5 nF ceramic | Compensation VC | Loop stability |
| D1 | MBRS330T3 (3A/30V) | Freewheel Diode | VF low, Schottky |
| D2 | 1N914 / 1N4148 | Diode BOOST | Fast, low VF |
| R1 | 4,32 kΩ 1 % | Divider FB (haut) | Set VOUT |
| R2 | 2,49 kΩ 1 % | Divider FB (bas) | Set VOUT |

# 11. Electrical Schematic

Figure 1 — Schematic of Buck LT1959 converter (7,4 V → 3,3 V / 200 mA)

# 12. Simulated waveforms

Figure 2 — Inductor current IL, voltage VSW and output voltage VOUT over 2 periods (f = 500 kHz)

The orang area represents the conduction time of the switch (ton ≈ 0,892 µs).

VOUT ripple is mainly due to the ESR of the capacitor C3 (≈ 36 mV < 50 mV spec)  ✓

Reference : Linear Technology — LT1959 Datasheet, Rev. A.

Calculus realized for IS = 200 mA nominal. Check thermal parameters for higher currents.